# Top Precision MEMS Accelerometers

## Analog Devices (ADI) – Industry Leader

### ADXL355/ADXL357 (Ultra-Low Noise)

- Noise density: 25 μg/√Hz (ADXL355), 80 μg/√Hz (ADXL357)

- Range: ±2g/±4g/±8g (ADXL355); ±10g/±20g/±40g (ADXL357)

- Bias instability: 0.15 mg (typ)

- Interface: SPI

- Price: $30–$80

- Applications: Seismic monitoring, inclinometry, industrial IoT

### ADXL100x Series (High Frequency, High-g)

- ADXL1001: ±100g, 24kHz bandwidth, 25 μg/√Hz

- ADXL1002: ±50g, 22kHz bandwidth, 30 μg/√Hz

- ADXL1004: ±500g, 24kHz bandwidth

- Price: $50–$150

- Applications: Condition monitoring, predictive maintenance

### ADXL354/ADXL356 (Military Grade)

- Qualification: MIL-PRF-38534 Class H

- Noise: 20 μg/√Hz

- Temperature range: -55°C to +125°C

## STMicroelectronics

### IIS3DWB (High Stability, Wide Bandwidth)

- Noise density: 75 μg/√Hz

- Bandwidth: 1.7kHz

- Range: ±2/±4/±8/±16g

- Interface: SPI/I²C

- Applications: Structural health monitoring, vibration analysis

### AIS2IH (Low Noise, High Temp)

- Noise: 90 μg/√Hz

- Temperature: -40°C to +105°C

- Shock resistant: 10,000g

## Bosch Sensortec

### BMA400 (Ultra-Low Power with Good Precision)

- Noise: 150 μg/√Hz

- Power: 14 μA at 100Hz

- Applications: Wearables with activity recognition

### SMA7x0 (High Performance)

- Noise: <100 μg/√Hz

- Applications: Automotive, industrial

### Kionix (Rohm)

### KX134-1211 (Wide Range, Low Noise)

- Range: ±8/±16/±32/±64g

- Noise: 100 μg/√Hz (±8g range)

- Bandwidth: 6.6kHz

- Price: $5–$15

## Colibrys (Swiss) – Ultra-High Precision

MS9000.D (Servo MEMS)

- Bias stability: <0.2 mg

- Noise: 0.5 μg/√Hz (ultra-low!)

- Scale factor stability: <100 ppm

- Price: $500–$2,000

- Applications: Inertial navigation, geophysics

SF3000L

- Range: ±2g

- Noise: 1.2 μg/√Hz

- Applications: Tilt sensing, seismic

Application-Specific Recommendations

1. Seismic/Very Low Frequency (<10Hz)

- Best: ADXL355, Colibrys MS9000, Silicon Designs 1221

- Critical: Ultra-low noise (<50 μg/√Hz), DC response

2. Structural Health Monitoring (1–1000Hz)

- Best: ADXL1001, ST IIS3DWB, PCB 333B30 (IEPE)

- Critical: Good noise performance, wide bandwidth

3. Inertial Navigation/Guidance

- Best: Honeywell QA-3000, Silicon Sensing CRS39, Analog Devices ADIS1649x (IMU)

- Critical: Bias stability (<1 mg), scale factor stability

4. Tilt/Inclinometry

- Best: ADXL357, Murata SCA830, Jewell Instruments

- Critical: Low temperature coefficient, good 0g stability

5. Condition Monitoring (Machinery)

- Best: ADXL1002, Wilcoxon 786A, Metrix Vibration Sensors

- Critical: High frequency response, ruggedness

6. Geophysical/Research

- Best: Güralp CMG-5T, Nanometrics Trillium, Streckeisen STS-2

- Critical: Sub-μg sensitivity, very low noise

Precision vs. Cost Trade-off

| Performance Tier | Noise Density | Bias Stability | Price Range | Example |
| --- | --- | --- | --- | --- |
| Consumer | >200 μg/√Hz | >10 mg | $1–$10 | MMA8451, LIS2DH |
| Industrial | 50–200 μg/√Hz | 1–10 mg | $10–$100 | ADXL345, IIS2DLPC |
| High Precision | 20–50 μg/√Hz | 0.1–1 mg | $30–$300 | ADXL355, IIS3DWB |
| Ultra-Precision | <20 μg/√Hz | <0.1 mg | $200–$2,000 | Colibrys, Honeywell servo |
| Research Grade | <1 μg/√Hz | <0.01 mg | $1,000–$20,000 | Güralp, Nanometrics |

## Critical Design Considerations

1. Signal Chain Requirements

- ADC resolution: 16–24 bits needed for precision

- Anti-aliasing filtering: Essential for vibration applications

- Digital filtering: Often needed to reduce noise further

2. Calibration Needs

- Factory calibration: Essential for precision parts

- Temperature compensation: Needed for <1 mg stability

- In-system calibration: May require periodic recalibration

3. Environmental Factors

- Temperature effects: Biggest source of error

- Mechanical mounting: Affects frequency response

- Cable noise/microphonics: Critical for low-level signals

## Emerging Technologies

1. Optical MEMS

- Companies: Si-Ware, SpectraMEMS

- Advantage: Immune to EMI, very high precision

2. Nano-g Accelerometers

- Companies: OEWaves, AOSense (atom interferometry)

- Performance: <1 ng/√Hz noise

- Applications: Gravity mapping, fundamental physics

3. Quartz MEMS Hybrid

- Companies: Vesper, Rakon

- Advantage: Excellent long-term stability

Where to Buy/Manufacturers

Commercial/Industrial

- Digi-Key/Mouser: ADI, ST, Kionix parts

- PCB Piezotronics: IEPE accelerometers

- TE Connectivity/Measurement Specialties

High-End/Research

- Colibrys (Switzerland)

- Honeywell Aerospace

- Silicon Designs

- Geophysical suppliers: Güralp, Nanometrics, Lennartz

OEM/Volume

- Direct from manufacturers with calibration services

Bottom line: For most precision applications, ADXL355/ADXL357 offer the best balance of performance and cost. For the highest precision, servo/force-balance or optical technologies are needed but at significantly higher cost and complexity.

## Specification Comparison

| Parameter | ADXL355 (Analog Devices) | IIS3DWB (STMicroelectronics) | Winner |
| --- | --- | --- | --- |
| Technology | 3-axis capacitive MEMS | 3-axis capacitive MEMS | Tie |
| Measurement Range | ±2g, ±4g, ±8g (programmable) | ±2g, ±4g, ±8g, ±16g | IIS3DWB (wider range) |
| Noise Density | 25 μg/√Hz (typ) | 75 μg/√Hz (typ) | ADXL355 (3× lower noise) |
| Bandwidth | 1 kHz (LPF configurable) | 1.7 kHz (fixed) | IIS3DWB (higher BW) |
| Output Data Rate | Up to 4 kHz | Up to 26.7 kHz | IIS3DWB (higher ODR) |
| Bias Instability | 0.15 mg (typ) | Not specified (est. 0.5–1 mg) | ADXL355 |
| Zero-g Offset | ±20 mg (max) | ±60 mg (max) | ADXL355 |
| Temperature Coefficient | 0.5 mg/°C (typ) | Not specified (est. 1–2 mg/°C) | ADXL355 |
| Non-linearity | 0.025% of FS | 0.1% of FS | ADXL355 |
| Interface | SPI only | SPI & I²C | IIS3DWB (more flexible) |
| FIFO | 512 samples | 3 kB | IIS3DWB (larger) |
| Power Consumption | 200 μA (ODR 4000 Hz) | 1.6 mA (ODR 26670 Hz) | ADXL355 (lower power) |
| Supply Voltage | 2.25–3.6 V | 1.71–3.6 V | IIS3DWB (wider range) |
| Temperature Range | -40°C to +125°C | -40°C to +105°C | ADXL355 (wider high end) |
| Shock Survivability | 10,000g | 10,000g | Tie |
| Package | 3.0×3.0×1.0 mm LGA | 2.5×3.0×0.83 mm LGA | IIS3DWB (smaller) |
| Price (1k units) | $25–$35 | $8–$12 | IIS3DWB (cheaper) |

Application Match

ADXL355 is better for:

- Low-frequency precision measurements (seismic, structural tilt)

- High-resolution DC measurements (inclinometers, leveling)

- Low-power precision monitoring

- Applications requiring temperature stability

- Sensor fusion in high-end IMUs (with gyros)

IIS3DWB is better for:

- Vibration analysis (condition monitoring)

- Higher frequency content (>500 Hz)

- Bandwidth-critical applications

- Cost-sensitive precision applications

- Space-constrained designs (smaller package)

Performance Visualization

text

Noise Performance Comparison:

ADXL355:  ████████████████ 25 μg/√Hz (Excellent for DC/low freq)

IIS3DWB:  ████████ 75 μg/√Hz (Good for vibration)

Bandwidth Comparison:

ADXL355:  ██████████ 1 kHz

IIS3DWB:  ████████████████ 1.7 kHz

Price Comparison:

ADXL355:  ████████████████ $30

IIS3DWB:  ██████ $10

Key Differentiators

ADXL355 Advantages:

- Superior noise performance (best in class for MEMS)

- Better DC characteristics (bias stability, temperature coefficient)

- Lower power consumption for given ODR

- Better documented performance (ADI provides full characterization)

- Military-grade versions available (ADXL356)

IIS3DWB Advantages:

- Higher bandwidth & ODR

- Lower cost (1/3 the price)

- Smaller package

- Dual interface (SPI + I²C)

- Larger FIFO (3 kB vs 512 B)

Typical Use Cases

Choose ADXL355 if you need:

- Structural health monitoring of buildings/bridges (low freq)

- Geophone replacement in seismic arrays

- Precision tilt sensing (<0.01° resolution)

- Inertial navigation (combined with high-end gyros)

- Scientific instrumentation

Choose IIS3DWB if you need:

- Machine condition monitoring (vibration analysis)

- Predictive maintenance systems

- Industrial IoT vibration sensors

- Shock detection in industrial equipment

- Cost-effective precision in consumer/industrial devices

Signal Chain Considerations

ADXL355:

- Requires good PCB layout (noise-sensitive)

- Benefits from external precision reference

- SPI interface only (needs MCU with SPI)

- Built-in temperature sensor for compensation

IIS3DWB:

- More layout-forgiving

- I²C option simplifies design

- Embedded pedometer/activity detection

- INT pins for interrupts/wake-up

Ecosystem & Support

ADXL355:

- Excellent documentation from ADI

- Evaluation boards available (~$200)

- Software drivers for various platforms

- Application notes for precision designs

- Simulation models (SPICE)

IIS3DWB:

- ST ecosystem (CubeMX, HAL drivers)

- Lower-cost evaluation boards (~$50)

- Arduino compatibility

- Wider microcontroller compatibility

Alternatives to Consider

Between these two in performance:

- ADXL357: ±10/20/40g version of ADXL355 (slightly higher noise)

- IIS2DLPC: Ultra-low power from ST (but lower performance)

- BMA400: Ultra-low power with decent performance (Bosch)

Higher performance (more expensive):

- Colibrys MS9000: Servo MEMS (0.5 μg/√Hz noise)

- Silicon Designs 1221: Precision quartz-based

- Honeywell QA-3000: Force-balance (highest precision)

Recommendation Summary

| Scenario | Recommendation | Reason |
| --- | --- | --- |
| Budget-constrained vibration monitoring | IIS3DWB | Good performance at 1/3 the cost |
| High-precision tilt/inclination | ADXL355 | Superior DC stability & noise |
| Industrial condition monitoring | IIS3DWB | Higher bandwidth, good enough noise |
| Seismic/low-frequency monitoring | ADXL355 | Best low-frequency noise performance |
| Battery-powered precision sensing | ADXL355 | Lower power consumption |
| High-volume consumer/industrial | IIS3DWB | Cost, size, interface advantages |
| Navigation/guidance systems | ADXL355 | Better bias stability for integration |

Bottom line:

- ADXL355 = Precision king for DC/low-frequency applications

- IIS3DWB = Best value for vibration/AC applications with bandwidth needs

If your application requires <100 Hz measurements with highest stability → ADXL355
If you need >100 Hz vibration analysis at lower cost → IIS3DWB

# Top Precision MEMS Gyroscopes

## 1. Analog Devices (ADI) – Market Leader

ADIS1649x Series (Tactical Grade)

- ADIS16490:

  - Bias instability: 6 °/h

  - ARW: 0.3 °/√h

  - Range: ±450 °/s

  - Price: ~$2,000

- ADIS16495:

  - Bias instability: 2.5 °/h

  - ARW: 0.1 °/√h

  - Integrated IMU (6-axis)

ADXRS453 (High Vibration Rejection)

- Bias stability: 12 °/h

- Vibration rejection: >1000 g's

- Applications: Industrial robotics, platform stabilization

## 2. STMicroelectronics

I3G4250D (Industrial Grade)

- Bias stability: 10 °/h

- ARW: 0.03 °/s/√Hz

- Range: ±245/±500/±2000 °/s

- Price: $5–$15

ISM330DHCX (6-axis IMU)

- Gyro noise: 2.5 mdps/√Hz

- Bias stability: Not specified (est. 20–50 °/h)

- Smart features: Machine learning core

## 3. Bosch Sensortec

BMG250 (Precision 3-axis)

- Noise: 2.5 mdps/√Hz

- Bias instability: <10 °/h (typ)

- Applications: Industrial automation, robotics

## 4. Sensonor (Infineon) – Best Performance

STIM210 (Tactical Grade Standard)

- Bias instability: 0.5 °/h (allan deviation)

- ARW: 0.15 °/√h

- Scale factor stability: 150 ppm

- Price: $3,000–$5,000

- Status: Industry benchmark for MEMS gyros

STIM300 (3-axis IMU)

- Gyro bias instability: 0.5 °/h

- ARW: 0.15 °/√h

- Temperature range: -40°C to +85°C

- Shock: 2000 g

- Applications: Aerospace, defense, land navigation

## 5. Silicon Sensing (Japan) – Resonant MEMS

CRS39 (Tactical Grade)

- Bias instability: 1 °/h

- ARW: 0.2 °/√h

- Technology: VSG (vibrating structure gyro)

- Applications: UAVs, robotics, industrial

CRS09 (High Performance)

- Bias instability: 0.3 °/h

- ARW: 0.05 °/√h

- g-sensitivity: <0.1 °/h/g

6. Epson (Japan) – Crystal MEMS

XV7000/8000 Series

- Bias stability: 10 °/h (XV7000), 3 °/h (XV8000)

- ARW: 0.05–0.2 °/√h

- Technology: QMEMS (quartz + MEMS)

- Applications: Automotive, industrial

## 7. Murata (Japan)

SCC2000 Series (3-axis gyro)

- Bias stability: 10–20 °/h

- ARW: 0.2 °/√h

- Applications: Industrial, robotics

## 8. Gladiator Technologies (US)

LandMark 30/100 Series

- Bias instability: 1–10 °/h

- ARW: 0.1–0.3 °/√h

- Applications: Military vehicles, marine

## Application-Specific Recommendations

1. Aerospace/Defense Navigation

- Best: Sensonor STIM210/300, Silicon Sensing CRS39

- Requirements: <1 °/h bias, low g-sensitivity, wide temperature

2. Industrial Robotics

- Best: ADIS16490, Bosch BMG250, Epson XV8000

- Requirements: 1–10 °/h, vibration resistant

3. Autonomous Vehicles

- Best: ADIS16495 (IMU), ST ISM330DHCX

- Requirements: Good ARW, moderate bias stability

4. Marine/Stabilization

- Best: Silicon Sensing CRS39, Gladiator LandMark

- Requirements: Good bias stability, moderate cost

5. Consumer/Prosumer

- Best: ST I3G4250D, Bosch BMG250

- Requirements: <$50, reasonable performance

Performance Comparison Table

| Model | Manufacturer | Bias Instability | ARW | Price | Best For |
| --- | --- | --- | --- | --- | --- |
| STIM300 | Sensonor | 0.5 °/h | 0.15 °/√h | $3,000–5,000 | Aerospace, defense |
| CRS39 | Silicon Sensing | 1 °/h | 0.2 °/√h | $1,000–2,000 | UAVs, robotics |
| ADIS16495 | Analog Devices | 2.5 °/h | 0.1 °/√h | $2,000–3,000 | Tactical navigation |
| XV8000BB | Epson | 3 °/h | 0.2 °/√h | $500–1,000 | Industrial automation |
| BMG250 | Bosch | <10 °/h | 0.01 °/s/√Hz | $20–50 | Consumer/industrial |
| I3G4250D | ST | 10 °/h | 0.03 °/s/√Hz | $5–15 | Low-cost industrial |

## Critical Design Considerations

1. Temperature Compensation

- Precision gyros require extensive temperature calibration

- Look for: Built-in temperature sensors, factory calibration coefficients

- Best practice: Implement 3rd-order polynomial compensation

2. Vibration/Environmental

- g-sensitivity: Critical for moving platforms

- Acoustic sensitivity: Some MEMS gyros pick up acoustic noise

- Shock resistance: Typically 1000–5000 g

3. Signal Processing

- Filtering needed: Most precision applications require 50–200 Hz bandwidth

- ADC resolution: 16–24 bits recommended

- Oversampling: Can improve noise performance

4. Calibration Requirements

- Multi-position calibration (6+ positions for gyrocompassing)

- Temperature cycling (-40°C to +85°C)

- Rate table calibration for scale factor

## Emerging Technologies

1. MEMS Coriolis Vibratory Gyros (CVG)

- Companies: InnaLabs, Atlantic Inertial

- Advantage: Better bias stability

2. Optical MEMS Gyros

- Companies: OEwaves, KVH (Fiber optic, not MEMS)

- Performance: Navigation grade in MEMS form factor

3. NEMS (Nano-electromechanical)

- Research stage: Sub-0.1 °/h bias potential

- Challenges: Manufacturing, packaging

4. Atomic Gyros (Not MEMS but compact)

- Companies: AOSense, Muquans

- Performance: <0.001 °/h bias

## Price vs. Performance Trade-off

text

Bias Stability vs Price:

<1 °/h:    ██████████████████ $3,000–10,000 (Sensonor, high-end ADI)

1–10 °/h:  ██████████ $500–3,000 (Silicon Sensing, Epson, mid ADI)

10–50 °/h: ███ $50–500 (ST, Bosch, Murata)

>50 °/h:   █ <$50 (Consumer MEMS)

## Manufacturer Comparison

| Company | Strength | Typical Bias | Price Range |
| --- | --- | --- | --- |
| Sensonor | Best performance | 0.1–1 °/h | $2,000–10,000 |
| Silicon Sensing | Good value | 1–5 °/h | $500–3,000 |
| Analog Devices | Complete solutions | 2–10 °/h | $1,000–5,000 |
| Epson | Industrial reliability | 3–20 °/h | $200–2,000 |
| STMicro | Volume production | 10–100 °/h | $5–100 |
| Bosch | Automotive grade | 10–50 °/h | $10–50 |

## Where to Buy

High Precision (Tactical/Navigation):

- Direct from manufacturers (Sensonor, Silicon Sensing, ADI)

- Authorized distributors: Digi-Key, Mouser (limited high-end)

- Specialized distributors: VectorNav, SBG Systems (integrated units)

Industrial Grade:

- Digi-Key/Mouser: ADI, ST, Epson, Murata

- LCSC: Chinese alternatives

Evaluation/Development:

- Evaluation kits: $200–$2,000

- IMU boards: From VectorNav, SBG, ACEINNA

## Selection Guide

For Navigation (<1 °/h required):

- Must use tactical grade (Sensonor STIM300, Silicon Sensing CRS39)

- Budget: $3,000+

For Stabilization (1–10 °/h):

- ADI ADIS1649x or Epson XV8000

- Budget: $500–$2,000

For Industrial Robotics (10–50 °/h):

- ST ISM330DHCX or Bosch BMG250

- Budget: $20–$200

For Consumer/Educational:

- ST I3G4250D or MPU6050 (gyro only)

- Budget: <$50

Bottom line:

- Best performance: Sensonor STIM300 (0.5 °/h bias)

- Best value: Silicon Sensing CRS39 (1 °/h bias at lower cost)

- Best ecosystem: Analog Devices ADIS1649x (full IMU, good support)

- Budget industrial: Epson XV8000 or ST high-end

For most applications needing <10 °/h bias stability, MEMS technology now rivals much more expensive FOG (Fiber Optic Gyro) systems at lower size, weight, and power.