# Table of Contents

IMU Calibration

IMU Calibration Routines

Common

Common (Auxilary) functions. Consist of some handy scripts for common tasks.

Coning and Sculling

Coning and Sculling Algorithms

Examples 102

Some miscellaneous navigation examples. For simple introductionary navigation examples look at the forward parts of the smoothers (e.g. smoother_2filt_fwd, bf_fwd etc). The examples in this section contain a little more than what you may require to write a simple INS.

IMU Error Modeling

Stochastic modeling of IMU short term (stability) errors. Contains scripts for both Allan variance and correlation based tools.

INS initialization

Routines for INS Initialization & Self Calibration

INS Implementations

Inertial navigation system implementations with different mechanization frames together with the corresponding system models

Old Versions

Depreciated scripts. They are not used any more. However, existing scripts may still call these functions

Path Generators

Path generators and IMU simulators

PDA Navigation

Basic navigation functions and utilities for smart phones

Smoother and Kalman Filter Implementations

Different smoother implementations. The forward parth of each example implements a different odometer, zupt and loosely coupled GPS aided INS with a Kalman filter. Some of the examples also demonstrates non-singular INS implementations for polar navigation.

Transformation Functions

Transformation functions and operators. These are used to transform a coordinate frame into another.

# IMU Calibration Routines

The following scripts demonstrate a very primitive calibration routine.

TODO:: I had better write a better set of calibration routines describing each element in the affine transformation matrix together ith the emphasis on temperature effects.

| cp_affine_v000.m | Compute the affine transformation (calibration) parameters |
| --- | --- |
| cp_sensorio_v000.m | Helper function for calibration. Used to determine the mean sensor output at each static position. |
| example_calib.m | Calibration example without temperature compensation |

# Common (Auxilary) functions

Auxilary functions which are used in different scripts.

| cp_SSgain_v000.m | Computes a steady state gain for unobservable and uncontrollable navigation error models |
| --- | --- |
| dc2dc_v000.m | Given a discrete/continuous time DSR, converts it to continuous/discrete time DSR |
| diagmat_v000.m | Adds sub-matrices diagonally |
| downsample_v000.m | Downsamples the (imu) data by averaging it. (Imu data must not simply decimated to reduce data rate) |
| imu_err_defs_v000.m | Previously, I was using mdl_xxx functions to be able to seperate IMU error models from navigation models. I no longer follow that approach. Instead, I am going to put all error models in a single file and use it like a header file inside sys_xxx functions. Therefore, the new sys_xxx functions becomes dependent on the imu error model definitions which makes them unmodular. However, I found this approach much simpler for the users of the toolkit. |
| imu_err_model_v000.m | A script which converts imu_models into a standard model that can be directly used by sys_model scripts. Similar to imu_err_defs this is also model dependent. Whenever you add a new model to imu_err_defs, you must also add a corresponding cenversion routine in this script. |
| interp_imu_v001.m | Interpolate and syncronize the raw data (especially required for PDAs) |
| lsmat_v001.m | least squares (or ML for gaussian) solutions for linear models |
| markov1st_v000.m | discrete to/from continuous conversion for AR(1) models |
| mat2vec_v000.m | Symmetric matrix to/from vector conversion for covariance matrices. Especially used to save intermediate variables in smoothers. |
| posdiff_v000.m | Computes the metric position difference given 2 geodetic coordinates (i.e. latitude, longtitude, height) or Cne matrix. |
| readbin_v001.m | Read binary files |
| skew.m | Given a vector, transforms it to a skew symmetric matrix |
| ss2corr_v000.m | Computes the correlation (matrix) sequence given a stable ss (A,B,C,D) model |
| unskew.m | Given skew symmetric matrix, transforms it to a vector |

# Coning and Sculling Algorithms

An implementation of Ignagni's optimal coning and sculling algorithms.

| coning_minor_v000.m | Coning correction in a minor interval |
| --- | --- |
| coning_v000.m | Total coning correction in a major interval consisting several minor intervals |
| cs_example.m | Just shows hot to call the functions offline |
| sculling_coning_v000.m | sculling and coning corrections computed with equivalency principle |
| sculling_minor_v000.m | Sculling correction in a minor interval |
| sculling_v000.m | Total sculling correction in a major interval consisting several minor intervals |

# Examples 102

These are some miscellaneous INS implementation examples. If you need loosely coupled GPS/INS examples, you should first look at the examples in the smoother section.

INS Implementation without heading initialization

A GPS INS implementation which can be used without heading initialization. 2 states are used in a Kalman filter to represent large heading angle uncertainty.

Redundant Sensor Demonstration

Some INS examples hich demonstrates the use of redundant inertial sensors.

Tightly coupled GPS INS

A tightly coupled GPS INS system which is mechanized in ECEF. The Kalman filter only processes the pseudoranges.

Wander Azimuth Implementation

A non-singular INS implementation mechanized in wander azimuth frame. Furhtermore, The Kalman filter is based on PSI model.

## Large Heading Example

| example_largeheading.m | An integrated navigation example which starts with an unknown large heading angle assumption. After sufficient position observations, the system switches to fine alignment and then standard small angle modes. |
| --- | --- |
| sys_path_largeheading.m | System definition for example_largeheading |

## Redundant Inertial Sensor Demonstration

| example_MultiINSvsSRIMU.m | A strictly theoretical example comparing multi-ins and single-INS approaches for SRIMU based systems. |
| --- | --- |
| example_OptimalFusion.m | An example showing the optimal fusion of inertial sensors for skew redundant inertial measurement units |
| example_SRIMUNav.m | An example showing the different methods of processing SRIMU data in a navigation system |

The following function are just simple helpers used in the above scripts

| cp_M_v000.m |  |
| --- | --- |
| cp_redsys_v000.m | Reduced order system model for redundant yet identical sensors. |
| cp_T_v000.m |  |
| st_size_v000.m |  |

## Tightly Coupled GPS INS Example

| Tightly_v000.m | Simple Tightly coupled GPS INS integration with ECEF mechanization. If you need a loosly coupled structure, just look at the forward part of the smoothers. |
| --- | --- |
| sys_def_v000.m | System Definition for Tightly_v000 example |

## Wander Azimuth INS implementation

| example_wander.m | A non-singular wander-azimuth implementation example. The strapdown and the Kalman filter in this example can be used for performing inertial navigation at the poles. This example also demonstrates how to implement PSI error model in a Kalman Filter. |
| --- | --- |

# Modeling Stochastic Errors of Inertial Sensors

If you have difficulties in understanding the specification sheet values of inertial sensors or obtaining error model parameters from the Allan variance curves, this introductory note on stochastic errors of inertial sensors may help you.

Check this link for an informal critique of Allan variance method.

allan2markov_v000.m	Given the (peak) value of the Allan standard deviation, computes the associated 1st order markov parameters. This may be used to approximate the flicker noise as AR(1).

arma2allan_v000.m	Computes the theoretical allan variance for arma (p,n) (p < n)(including rw) models. You may use this functin to verify how close your approximate markovian error models is to the actual (observed) Allan variance.

arma2cov_v000.m	Given an arma model, computes the exponential coefficients of theoretical correlation of the model. (You must note that the correlation of arma models are always in the form of exponential functions)

cp_allan_var_v000.m	Computes the allan variance of the inertial sensor outputs. Most probably this is the only function that you may need in this section. You can extract the stochastic error model coefficients from the AV figure computed by this script.

cp_wn_pow_v000.m	computes the disturbance power of the random walk and white noise processes.

Example_conf.m	An example script which demonstrates the use of modeling functions in this section.

exp2markov_v000.m	A helper function for the example script

exp_approx_v000.m	Given some exponential functions, computes a single approximate exponential. (Helper function for the example)

exp_approx1_v000.m	Similar to exp_approx

exp_approx2_v000.m	Similar to exp_approx

exp_approx3_v000.m	Similar to exp_approx

expo_fit_v000.m	Given the autocorrelation data, fits an exponential function

lvl_filter_v000.m	DCT based Haar filter implementation. (See the example for the use of this function in correlation computations)

# INS Initialization & Self Calibration

Proper initialization of attitude states is in fact the toughest problem in any INS design task. Most people simply assume that the INS can be started with some arbitrary external attitude information. However such an over-approximation may cause serious complications.

The following scprits demonstrates a couple of ways to determine the initial orientation based on inertial sensors. Self-calibration examples further demonstrates the use of redundant sensors and ZUPTs to estimate sensor errors during stationary instants

| align_opt_v000.m | Algebraic and SVD based attitude solution from vectors |
| --- | --- |
| align_wander_v000.m | Platform leveling using a rotation vector |
| alingc_2s_v000.m | 2 step IMU Coarse Align Method (first platform, then heading) |
| alingc_hd_v000.m | Coarse Align: Heading |
| alingc_pl_v000.m | Coarse Align: Platform |
| alingc_std_v001.m | Standard Coarse Align Procedure |
| example_align.m | Usage Examples for coarse align methods |
| example_selfcalib.m | Usage example for self calibration |
| selfcalib1_v001.m | Stationarity based self calibration using Kalman filter |
| selfcalib2_v000.m | Multi source based self calibration using Kalman filter |
| sys_def.m | system definitions for above examples. |

# INS Implementations

This sections contains ins implementations mechanized in different frames with different attitude represenations. Furthermore, system models representing the propagation of errors on the corresponding INS mechanizations are also presentented.

| correctnav_Cen_v000.m | Corrects the navigation states (i.e. position, velocity and attitude) using the Kalman generated error values. This function can be used for both PHI and PSI formulated Kalman filters. |
| --- | --- |
| geoparam_v000.m | Re, Rn, G etc. for NED mechanization. |
| geoparam_v001.m | Computes the curvature matrix (Elipsoidal radious of Earth) and the gravity. This is a non-singular implementation which must be used for polar navigation. |
| la_imu_v000.m | lever arm computations INS+IMU |
| la_sen_v000.m | Lever arm computations for sensor outputs only (subset of la_imu) |
| mdl_ecef_dcm_v000.m | Wrapper for the sys_ecef_dcm (TO BE DEPRECIATED) (See:imu_err_defs) |
| mdl_ned_dcm_v000.m | Wrapper for the sys_ned_dcm (this is called by the KF, this implicity calls sys_ned_dcm) (TO BE DEPRECIATED) (See:imu_err_defs) |
| strapdown_bckwrd_Cen_quat_v000.m | Backward implementation of strapdown_Cen_quat (used in 2-filter smoothers) |
| strapdown_bd_dcm_v000.m | Body frame mechanization of strapdown equations with DCM |
| strapdown_Cen_dcm_v000.m | Wander-Azimuth implementation with position defined as Cen (singular) |
| strapdown_Cen_quat_v000.m | Wander-Azimuth implementation with position defined as Cen and attitude mechanized as quaternion |
| strapdown_ecef_dcm_v000.m | ECEF frame mechanization of strapdown equations with DCM |
| strapdown_ecef_quat_v000.m | ECEF frame mechanization of strapdown equations with quaternions |
| strapdown_ned_dcm_v000.m | NED frame mechanization of strapdown equations with DCM |
| strapdown_ned_quat_v000.m | NED frame mechanization of strapdown equations with quaternions |
| strapdown_pln_dcm_v000.m | Navigation in Planar Non-rotating Earth |
| strapdown_wander_dcm_v000.m | Wander-Azimuth implementation (Singular) |
| strapdown_wander_quat_v000.m | Wander-Azimuth implementation with quaternions (Non-Singular) |
| sys_bd_dcm_v000.m | error propagation model for strapdown_bd_dcm |
| sys_ecef_dcm_v000.m | error propagation model for sys_ecef_dcm_v000 |
| sys_llh_phipsi_v000.m | System model for geodetic navigation frame mechanization for both phi and psi parametrization. This model can be used for both DCM and quaternion mechanizations. (e.g. strapdown_ned_quat_v000, strapdown_ned_dcm_v000). Position errors are modelled as delta_llh. - The phi model of this script is equivalent to sys_ned_dcm_v000. - The only difference between this and sys_metric_phipsi_v000 is the position error parameterization. |
| sys_metric_phipsi_v000.m | System model for geodetic navigation frame mechanization for both phi and psi parametrization. This model can be used for both DCM and quaternion mechanizations. (e.g. strapdown_Cen_dcm_v000, strapdown_Cen_quat_v000, strapdown_ned_dcm_v000). Position errors are modelled as delta_x (metric). |
| sys_ned_dcm_v000.m | error propagation model for strapdown_ned_dcm |
| sys_ned_dcm_v001.m | Similar to sys_ned_dcm_v000. However, this is modified for imu_err_defs usage. |
| sys_pln_dcm_v000.m | error propagation model for strapdown_pln_dcm |
| sys_wander_dcm_v000.m | System model for wander azimuth implementation (for strapdown_wander_dcm) |
| sys_wander_largeheading_v000.m | System model for large heading uncertainties (for strapdown_wander_dcm) |
| sys_wander_psi_v000.m | System model for wander azimuth implementation (for strapdown_wander_quat and strapdown_Cen_dcm) |

# Path Generators

You can use the folloing scripts to simulate IMU outputs on a trajectory. You can then process those simulated sensor outputs with the INS implementations (and examples) included in the toolkit.

# Basic Navigation Functions and utlities for SmartPhones

- Click here to see the Allan variance curves for the gyropscopes in the Nexus-S

| example_AttRef.m | Computes the attitude by optimally combining the accelerometer and gyroscope outputs with a Kalman filter. |
| --- | --- |
| AKMF | Open source akmd (inertial sensor and compass daemon) for HTC Hero. Check this page for the development notes. |
| iLogger | Inertial sensor data logger for iPhone and iPod touch. Check this page for some notes about the sensor subsystem of iOS |
| DataMonitor | A sensor and position manager monitor for android. Use this to convert sensor.bin to ascii. |
| Demo6DoF | An aided inertial navigation example for Android devices. Check this page for some notes about both the application and Android's sensor subsystem. |
| AndroLogger | A sensor logger application for android phones. The application sends the sensor data over the network using a udp socket. The associated linux program can be used to record these data packages on the PC. The program creates a new file for each different IP. Therefore, it can be used to record multi-sources at the same time. |

| AddIMUErr_v002.m | Adds the imu errors to the path gen outputs. (AddIMUErr_v001.m is now depreciated.) |
| --- | --- |
| AddObsErr_v000.m | Add observation errors to the measurements |
| flickgen_v000.m | Flicker noise generator for monte carlo analysis. |
| PathGen_v002.m | Main Path generator program. This script is capable of generating multi-imu/sensor outputs (together with true position/velocity/attitude values) given a motion definition. You can test other functions in the toolkit using the sensor outputs generated with this scripts. |
| PathGen_v003.m | A simplified version of PathGen_v002 for only a single INS (no redundancy, no vibration model etc). This can also simulate bank-to-turns for aircrafts. |

# Smoothers and Kalman Filters

The scripts in this section demonstrate GPS, odometer zupt aided INS implementations. Each of the example is in fact a complete smoother implementation. However, the forward parts can be used as a standard Kalman filtering solution for aided INS systems.

2 filters Smoother

This is a 2 filter smooter implementation. The backward part uses a backward INS to compute the reverse solution and a information form Kalman filter to compute the ML estimates.
You may use the forward filter as an example of a simple vehicular navigation system with GPS+odometer+Zupt. (The INS is mechanized in geodetic frame with quaternions. The position is updated using Cen. The system model is derived based on PSI-Formulation. The position errors are defined in meters rather than radians.)

Bryson-Frazier smoother

Standard Bryson-Frazier smoother implementation. The forward parth demonstrates an example of a aided INS for vehicles (It is a quaternion based INS which is mechanized in geodetic frame. The position is defined in lat, lon, and height. The error model is derived based on PHI formulation.)

Fixed Lag Smoother

This is a fixed lag smoother based on moving window innovation approach. (That is why, it is somehow close to BF formula). In order to deal with instability problem it uses 3 INSs simultaneously.

Rauch-Tung-Striebel Smoother

An RTS implementation which processes real Sensor data with GPS, Odometer and ZUPT. You may use the forward path as an example for a simple vehicular INS with GPS+Odometer+Zupt.

Weinert Smoother

This smoother is based on complementery space approach of Weinert. This specific example processes only ZUPT instants to self calibrate the IMU data without running any INS. The self-calibrated IMU data can later be processed by any other smoother/filter. This example also derives the new time-varying error model of the generated (self-calibrated) data.

## 2 Filters Smoother

| smoother_2filt_fwd.m | Forward filter and INS. Use this forward filter solution as a typical example of a Kalman filter with a PSI-model propagation model implementation. |
| --- | --- |
| smoother_2filt_bckwd.m | Backward filter and backward INS. The filter is in information form (the inverse of the initial covariance is taken to be 0) |
| combine_2filt_v000.m | Script to combine forward and backward solutions. |
| combine_2filt_v001.m | Script to combine forward and backward solutions (This method is suggested in Maybeck. However, I prefer the previous solution (v000) as it is easier to implement Cholesky in that script.) |

## Bryson-Frazier Smoother

Example I:

| Basic BF Smoother example which processes simulated data. |  |
| --- | --- |
| smoother_BF_frwd.m | Forward filter and INS. Use this forward filter solution as a typical example of a Kalman filter with a PSI-model propagation model implementation. |
| smoother_BF_bkwd.m | Bryson-Frazier smoother : Backward (Smoother part) |
| sys_path.m | Auxilary function to generate simulated data |

Example II

| Standard BF implementation which processes real Sensor data with GPS, Odometer and ZUPT. Use the forward path as an example for a simple vehicular INS (quaternion based INS mechanized in geodetic frame. The position is defined in lat, lon, and height. The error model is derived based on PHI formulation.) |  |
| --- | --- |
| bf_fwd.m | Forward Path (Standard Filtering Solution). Note that although the Strapdown is mechanized with quaternions the system model still uses 3 small angles to represents errors on attitude. |
| bf_bckwd.m | Backward Path (Standard BF Implementation) |
| klmn_scrpt.m | A simple script just to avoid code repetations |

## Fixed Lag Smoother

This is a stable fixed lag smoother implementation. It computes the smoothed estimates with a user-defined delay time. It uses INSs simultaneously in order not to maintain a buffer.

| smoother_FLS.m | Fixed Lag smoother. (Given a delay, it computes the smoothed estimates) |
| --- | --- |
| sys_path.m | Auxilary function to generate simulated data |

## Bryson-Frazier Smoother

Example I:

| Basic RTS Smoother example which processes simulated data. Instead of the the exact RTS form, this example implements a recursion very similar to BF. Compare this implementation with the Basic BF Smoother example. |  |
| --- | --- |
| smoother_RTS_frwd.m | RTS smoother : Forward (Filtering part) |
| smoother_RTS_bkwd.m | RTS smoother : Backward (Smoother part) |
| sys_path.m | Auxilary function to generate simulated data |

Example II

| An RTS implementation which processes real Sensor data with GPS, Odometer and ZUPT. You may use the forward path as an example for a simple vehicular INS with GPS+Odometer+Zupt. |  |
| --- | --- |
| mtd1 and mtd2 are 2 different backward RTS recursions. You may use any one of them. See the comments in the code to learn the difference between these recursions. |  |
| rts_fwd.m | Forward Path (Standard Filtering Solution). Compare this with the forward path of BF to see the difference between the recorded parameters. |
| rts_bckwd_mtd1.m | Backward Path Method 1: The backward corrections are accumulated in a different register. |
| rts_bckwd_mtd2.m | Backward Path Method 2: The backward corrections are applied directly to the smoothed navigation registers. This implementation is generally regarded as standard RTS solution. However, I generally prefer the solution of the previous example (the simulated RTS example) |
| klmn_scrpt.m | A simple script just to avoid code repetations |

## Weinert Smoother

| PreSmooth1.m | IMU pre-smoother based on Weinert's Smoother. Computes both the smoothed sensor output and associated forward time Markov model parameters |
| --- | --- |

# Orientation and Coordinate Frame Transformations

| dcm2euler_v000.m | DCM to Euler Angles |
| --- | --- |
| dcm2heading_v000.m | computes yaw angle from dcm |
| dcm2llh_v000.m | Cne (Earth to navigation frame DCM) to latitude, longitude, wander angle |
| dcm2quat_v000.m | Direction Cosine Matrix (DCM) to Quaternion |
| dcm2rot_v000.m | DCM to Rotation Vector |
| dir2euler_v000.m | Given a direction vector, computes the orientation (assumes roll=0) (e.g. Given the difference of 2 succesive GPS coordinates, computes the orientation of the vehicle assumung roll=0) |
| ecef2geo_v000.m | Converts ECEF (Earth centered Earth Fixed) position to Llh (Geodetic Position) and vice versa |
| euler2dcm_v000.m | Transforms Euler angles to DCM |
| llh2dcm_v000.m | Given latitude and longtitude and wander angle, computes Cen tranformation matrix (Navigation frame to ECEF DCM) |
| quat2dcm_v000.m | Convert quaternion into DCM |
| quat2rot_v000.m | Quaternion to Rotation Vector |
| quatmult_v001.m | Quaternion multiplication |
| quatrot_v000.m | Vector transformation using quaternions (rotate a vector given a quaternion) |
| rot2dcm_v000.m | Rotation vector to direction cosine matrix |
| rvec2quat_v000.m | Rotation vector to quaternion |